JASMIN AND ROSE DE MAI 1644370

Safety Data Sheet

according to the REACH Regulation (EC) 1907/2006 amended by Regulation (EU) 2020/878

Issue date: 10/14/2024   Version: 1.0

| SECTION 1: Identification of the substance/mixture and of the company/undertaking |  |
| --- | --- |
| 1.1. Product identifier |  |
| Product form | : Mixture |
| Trade name | : JASMIN AND ROSE DE MAI 1644370 |
| UFI | : 0G2W-8Q3Q-U408-NJJ6 |
| 1.2. Relevant identified uses of the substance or mixture and uses advised against |  |

Relevant identified uses

Main use category 	: Industrial use,Professional use,Consumer use

# 1.3. Details of the supplier of the safety data sheet

JPSHOP EURL

Le Chatel Route de Provins

77370 NANGIS

FRANCE

T +33(0)9 52 00 10 60 contact@jpshop.fr

# 1.4. Emergency telephone number

| Country/Area | Organisation/Company | Address | Emergency number | Comment |
| --- | --- | --- | --- | --- |
| France | ORFILA |  | +33 1 45 42 59 59 | This number provides contact details for all French Poison Control centers. These poison and toxicovigilance centers provide free medical assistance (excluding call costs), 24 hours a day, 7 days a week. |

# SECTION 2: Hazards identification

## 2.1. Classification of the substance or mixture

### Classification according to Regulation (EC) No. 1272/2008 [CLP]

Serious eye damage/eye irritation, Category 2 	H319

Skin sensitisation, Category 1 	H317

Hazardous to the aquatic environment – Chronic Hazard, 	H412

Category 3

Full text of H- and EUH-statements: see section 16

Adverse physicochemical, human health and environmental effects

May cause an allergic skin reaction. Causes serious eye irritation. Harmful to aquatic life with long lasting effects.

## 2.2. Label elements

### Labelling according to Regulation (EC) No. 1272/2008 [CLP]

Hazard pictograms (CLP) 	:

GHS07

| Signal word (CLP) | : | Warning |
| --- | --- | --- |
| Contains | : | 3,7-dimethylocta-2,6-dien-1-ol; cis-3,7-Dimethyl-2,6-octadien-1-ol; 3,7-Dimethyl octa-1,6diene-3-yl acetate; 3-Methyl-4-(2,6,6-trimethyl-2-cyclohexenyl)-3-buten-2-one; 4-tertButylcyclohexyl acetate; 4,7,7-trimethyl-8-oxabicyclo[2.2.2]octane; 1-(2,6,6-Trimethyl-3cyclohexenyl)-2-buten-1-one; 3,7-Dimethyloctan-3-ol; Coumarin; 1-(1,2,3,4,5,6,7,8Octahydro-2,3,8,8-tetramethyl-2-naphthyl)ethan-1-one; 3-(p-Methoxyphenyl)-2- methylpropionaldehyde; 2,4-dimethylcyclohex-3-ene-1-carbaldehyde; 2-Benzylideneoctanal; 3,7-Dimethyl octa-1,6-diene-3-ol; Benzyl Salicylate |
| Hazard statements (CLP) | : | H317 - May cause an allergic skin reaction. H319 - Causes serious eye irritation. H412 - Harmful to aquatic life with long lasting effects. |
| Precautionary statements (CLP) | : | P261 - Avoid breathing dust/fume/gas/mist/vapours/spray. P280 - Wear protective gloves/protective clothing/eye protection/face protection/hearing protection. P333+P313 - If skin irritation or rash occurs: Get medical advice/attention. P337+P313 - If eye irritation persists: Get medical advice/attention. P362+P364 - Take off contaminated clothing and wash it before reuse. |
| 2.3. Other hazards |  |  |

Contains no PBT and/or vPvB substances ≥ 0.1% assessed in accordance with REACH Annex XIII

The mixture does not contain substance(s) included in the list established in accordance with Article 59(1) of REACH for having endocrine disrupting properties, or substance(s) are not identified as having endocrine disrupting properties in accordance with the criteria set out in Commission

Delegated Regulation (EU) 2017/2100 or Commission Regulation (EU) 2018/605 at a concentration equal to or greater than 0,1 %

# SECTION 3: Composition/information on ingredients

## 3.2. Mixtures

| Name | Product identifier | % | Classification according to Regulation (EC) No. 1272/2008 [CLP] |
| --- | --- | --- | --- |
| 3-ethoxy-4-hydroxybenzaldehyde | CAS-No.: 121-32-4 EC-No.: 204-464-7 REACH-no: 01-211995896124 | 1 – 5 | Eye Irrit. 2, H319 |
| Benzyl Acetate | CAS-No.: 140-11-4 EC-No.: 205-399-7 REACH-no: 01-211963827242 | 1 – 5 | Aquatic Chronic 3, H412 |
| benzyl salicylate | CAS-No.: 118-58-1 EC-No.: 204-262-9 EC Index-No.: 607-754-00-5 REACH-no: 01-211996944231 | 1 – 5 | Eye Irrit. 2, H319 Skin Sens. 1B, H317 Aquatic Chronic 3, H412 |
| Tetrahydro-4-methyl-2-(2-methylpropyl)-2H-pyran-4-ol | CAS-No.: 63500-71-0 EC-No.: 405-040-6 REACH-no: 01-000001545864 | 1 – 5 | Eye Irrit. 2, H319 |
| 3,7-Dimethyl octa-1,6-diene-3-ol | CAS-No.: 78-70-6 EC-No.: 201-134-4 REACH-no: 01-211947401642 | 1 – 5 | Skin Irrit. 2, H315 Eye Irrit. 2, H319 Skin Sens. 1B, H317 |
| 2-tert-Butylcyclohexyl acetate | CAS-No.: 88-41-5 EC-No.: 201-828-7 REACH-no: 01-211997071333 | 1 – 5 | Aquatic Chronic 2, H411 |

| Name | Product identifier | % | Classification according to Regulation (EC) No. 1272/2008 [CLP] |
| --- | --- | --- | --- |
| Undecan-4-olide | CAS-No.: 104-67-6 EC-No.: 203-225-4 REACH-no: 01-211995933334 | 1 – 5 | Aquatic Chronic 3, H412 |
| geraniol; (2E)-3,7-dimethylocta-2,6-dien-1-ol | CAS-No.: 106-24-1 EC-No.: 203-377-1 EC Index-No.: 603-241-00-5 | 1 – 5 | Skin Irrit. 2, H315 Eye Dam. 1, H318 Skin Sens. 1, H317 |
| Oxacyclohexadecen-2-one | CAS-No.: 34902-57-3 EC-No.: 422-320-3 EC Index-No.: 606-092-00-4 REACH-no: 01-000001688362 | 1 – 5 | Aquatic Acute 1, H400 Aquatic Chronic 1, H410 EUH208 |
| 2-Benzylideneoctanal | CAS-No.: 101-86-0 EC-No.: 639-566-4 REACH-no: 01-211953309250 | 1 – 5 | Skin Sens. 1B, H317 Aquatic Acute 1, H400 Aquatic Chronic 2, H411 |
| Reaction mass of 3,5-dimethylcyclohex-3-ene-1carbaldehyde and 2,4-dimethylcyclohex-3-ene-1carbaldehyde | CAS-No.: 68039-49-6 EC-No.: 943-728-2 REACH-no: 01-211998238428 | 0.1 – 1 | Skin Irrit. 2, H315 Skin Sens. 1B, H317 Aquatic Chronic 2, H411 |
| 3-(p-Methoxyphenyl)-2-methylpropionaldehyde | CAS-No.: 5462-06-6 EC-No.: 226-749-5 REACH-no: 01-212062910367 | 0.1 – 1 | Skin Sens. 1B, H317 |
| 3-Methyl-4-(2,6,6-trimethyl-2-cyclohexenyl)-3-buten-2one | CAS-No.: 127-51-5 EC-No.: 204-846-3 | 0.1 – 1 | Skin Irrit. 2, H315 Aquatic Chronic 2, H411 Eye Irrit. 2, H319 Skin Sens. 1B, H317 |
| 3,7-Dimethyl octa-1,6-diene-3-yl acetate | CAS-No.: 115-95-7 EC-No.: 204-116-4 | 0.1 – 1 | Skin Irrit. 2, H315 Eye Irrit. 2, H319 Skin Sens. 1B, H317 |
| Coumarin | CAS-No.: 91-64-5 EC-No.: 202-086-7 REACH-no: 01-211994375626 | 0.1 – 1 | Acute Tox. 4 (Oral), H302 (ATE=500 mg/kg bodyweight) Skin Sens. 1B, H317 Aquatic Chronic 3, H412 |
| 1-(1,2,3,4,5,6,7,8-Octahydro-2,3,8,8-tetramethyl-2naphthyl)ethan-1-one | CAS-No.: 54464-57-2 EC-No.: 259-174-3 REACH-no: 01-211948998904 | 0.1 – 1 | Skin Irrit. 2, H315 Skin Sens. 1B, H317 Aquatic Chronic 2, H411 |
| 3,7-Dimethyloctan-3-ol | CAS-No.: 78-69-3 EC-No.: 201-133-9 REACH-no: 01-211945478821 | 0.1 – 1 | Skin Irrit. 2, H315 Eye Irrit. 2, H319 Skin Sens. 1B, H317 |
| 1-(2,6,6-Trimethyl-3-cyclohexenyl)-2-buten-1-one | CAS-No.: 57378-68-4 EC-No.: 260-709-8 REACH-no: 01-211953512253 | 0.1 – 1 | Acute Tox. 4 (Oral), H302 (ATE=1400 mg/kg bodyweight) Skin Irrit. 2, H315 Skin Sens. 1A, H317 Aquatic Acute 1, H400 Aquatic Chronic 1, H410 |

| Name | Product identifier | % | Classification according to Regulation (EC) No. 1272/2008 [CLP] |
| --- | --- | --- | --- |
| 4,7,7-trimethyl-8-oxabicyclo[2.2.2]octane | CAS-No.: 470-82-6 EC-No.: 207-431-5 REACH-no: 01-211996777224 | 0.1 – 1 | Flam. Liq. 3, H226 Skin Sens. 1B, H317 |
| cis-3,7-Dimethyl-2,6-octadien-1-ol | CAS-No.: 106-25-2 EC-No.: 203-378-7 | 0.1 – 1 | Skin Irrit. 2, H315 Eye Irrit. 2, H319 Skin Sens. 1B, H317 |
|  | CAS-No.: 1637294-12-2 EC-No.: 811-285-3 | 0.1 – 1 | Acute Tox. 4 (Inhalation), H332 (ATE=11 mg/l/4h) Skin Irrit. 2, H315 Eye Irrit. 2, H319 Skin Sens. 1B, H317 Aquatic Chronic 2, H411 |
| (Z)-3-Hexenyl 2-hydroxybenzoate | CAS-No.: 65405-77-8 EC-No.: 265-745-8 REACH-no: 01-211998732037 | 0.1 – 1 | Aquatic Acute 1, H400 Aquatic Chronic 2, H411 |
| 4-tert-Butylcyclohexyl acetate | CAS-No.: 32210-23-4 EC-No.: 250-954-9 REACH-no: 01-211997628624 | 0.1 – 1 | Skin Sens. 1B, H317 |

Full text of H- and EUH-statements: see section 16

| SECTION 4: First aid measures |  |
| --- | --- |
| 4.1. Description of first aid measures |  |
| First-aid measures general | : If you feel unwell, seek medical advice. |
| First-aid measures after inhalation | : Remove person to fresh air and keep comfortable for breathing. |
| First-aid measures after skin contact | : Wash skin with plenty of water. Take off contaminated clothing. If skin irritation or rash occurs: Get medical advice/attention. |
| First-aid measures after eye contact | : Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. If eye irritation persists: Get medical advice/attention. |
| First-aid measures after ingestion | : Call a poison center or a doctor if you feel unwell. |
| First-aid measures for first aider | : First aid workers will be equipped with suitable personal protective equipment. |
| 4.2. Most important symptoms and effects, both acute and delayed |  |
| Symptoms/effects after inhalation | : Although no appropriate human or animal health effects data are known to exist, this material is expected to be an inhalation hazard. |
| Symptoms/effects after skin contact | : May cause an allergic skin reaction. |
| Symptoms/effects after eye contact | : Eye irritation. |
| Symptoms/effects after ingestion | : None under normal conditions. |
| 4.3. Indication of any immediate medical attention and special treatment needed |  |

Treat symptomatically.

# SECTION 5: Firefighting measures

## 5.1. Extinguishing media

| Suitable extinguishing media | : Water spray. Dry powder. Foam. Carbon dioxide. |
| --- | --- |
| Unsuitable extinguishing media | : Do not use a heavy water stream. |

## 5.2. Special hazards arising from the substance or mixture

Fire hazard 	: No fire hazard.

Explosion hazard 	: No direct explosion hazard.

Hazardous decomposition products in case of fire 	: Toxic fumes may be released.

## 5.3. Advice for firefighters

Firefighting instructions 	: Fight fire from safe distance and protected location. Do not enter fire area without proper protective equipment, including respiratory protection.

Protection during firefighting 	: Do not attempt to take action without suitable protective equipment. Self-contained breathing apparatus. Complete protective clothing.

# SECTION 6: Accidental release measures

| 6.1. Personal precautions, protective equipment and emergency procedures |  |  |
| --- | --- | --- |
| General measures | : Stop leak if safe to do so. Notify authorities if product enters sewers or public waters. Absorb spillage to prevent material damage. |  |
| For non-emergency personnel Protective equipment | : Wear recommended personal protective equipment. |  |
| Emergency procedures | : Ventilate spillage area. Avoid contact with skin and eyes. Avoid breathing dust/fume/gas/mist/vapours/spray. |  |
| For emergency responders Protective equipment | : Do not attempt to take action without suitable protective equipment. For further information refer to section 8: "Exposure controls/personal protection". |  |
| Emergency procedures | : Evacuate unnecessary personnel. Stop leak if safe to do so. |  |
| 6.2. Environmental precautions |  |  |
| Avoid release to the environment. |  |  |
| 6.3. Methods and material for containment and cleaning up |  |  |
| For containment | : | Absorb spilled material with sand or earth. Contain any spills with dikes or absorbents to prevent migration and entry into sewers or streams. Stop leak without risks if possible. |
| Methods for cleaning up | : | Take up liquid spill into absorbent material. |
| Other information | : | Dispose of materials or solid residues at an authorized site. |
| 6.4. Reference to other sections |  |  |
| For further information refer to section 13. |  |  |
| SECTION 7: Handling and storage |  |  |
| 7.1. Precautions for safe handling |  |  |
| Additional hazards when processed | : | Not expected to present a significant hazard under anticipated conditions of normal use. |
| Precautions for safe handling | : | Ensure good ventilation of the work station. Avoid contact with skin and eyes. Avoid breathing dust/fume/gas/mist/vapours/spray. Wear personal protective equipment. |
| Hygiene measures | : | Contaminated work clothing should not be allowed out of the workplace. Wash contaminated clothing before reuse. Do not eat, drink or smoke when using this product. Always wash hands after handling the product. |
| 7.2. Conditions for safe storage, including any incompatibilities |  |  |
| Technical measures | : Keep in a cool, well-ventilated place away from heat. |  |
| Storage conditions | : Keep cool. Protect from sunlight. |  |
| Packaging materials | : Store always product in container of same material as original container. |  |
| 7.3. Specific end use(s) |  |  |
| No additional information available |  |  |

# SECTION 8: Exposure controls/personal protection

8.1. Control parameters

No additional information available

## 8.2. Exposure controls

### Appropriate engineering controls

Appropriate engineering controls:

Ensure good ventilation of the work station.

### Personal protection equipment

Personal protective equipment:

Wear recommended personal protective equipment. Personal protective equipment symbol(s):

### Eye and face protection

Eye protection: Safety glasses

### Skin protection

Skin and body protection:

Wear suitable protective clothing

Hand protection: Protective gloves

### Respiratory protection

Respiratory protection:

In case of insufficient ventilation, wear suitable respiratory equipment

### Environmental exposure controls

Environmental exposure controls: Avoid release to the environment.

# SECTION 9: Physical and chemical properties

## 9.1. Information on basic physical and chemical properties

Physical state 	: Liquid

Colour 	: colourless to yellow. Appearance 	: Liquid.

Odour 	: Not available

Odour threshold 	: Not available

Melting point 	: Not applicable

Freezing point 	: Not available Boiling point 	: Not available Flammability 	: Non flammable.

Lower explosion limit 	: Not available

Upper explosion limit 	: Not available

Flash point 	: > 60 °C (closed cup)

Auto-ignition temperature 	: Not available

Decomposition temperature 	: Not available pH 	: Not available

Viscosity, kinematic 	: Not available

Solubility 	: Not available Partition coefficient n-octanol/water (Log Kow) : Not available

| Vapour pressure | : Not available |  |
| --- | --- | --- |
| Vapour pressure at 50°C | : Not available |  |
| Density | : Not available |  |
| Relative density | : Not available |  |
| Relative vapour density at 20°C | : 0.914 – 0.924 |  |
| Particle characteristics | : Not applicable |  |
| 9.2. Other information |  |  |  |
| Other safety characteristics Refractive index | : | 1.454 – 1.464 |  |
| SECTION 10: Stability and reactivity |  |  |  |
| 10.1. Reactivity |  |  |  |

The product is non-reactive under normal conditions of use, storage and transport.

10.2. Chemical stability

Stable under normal conditions.

10.3. Possibility of hazardous reactions

No dangerous reactions known under normal conditions of use.

10.4. Conditions to avoid

None under recommended storage and handling conditions (see section 7).

10.5. Incompatible materials

No additional information available

10.6. Hazardous decomposition products

Under normal conditions of storage and use, hazardous decomposition products should not be produced.

# SECTION 11: Toxicological information

## 11.1. Information on hazard classes as defined in Regulation (EC) No 1272/2008

Acute toxicity (oral) 	: Not classified

Acute toxicity (dermal) 	: Not classified

Acute toxicity (inhalation) 	: Not classified

| geraniol; (2E)-3,7-dimethylocta-2,6-dien-1-ol (106-24-1) |  |
| --- | --- |
| LD50 oral rat | 3600 mg/kg |
| LD50 oral | 3600 mg/kg |
| LD50 dermal rabbit | > 5000 mg/kg Source: IUCLID,NLM,THOMSON |

| cis-3,7-Dimethyl-2,6-octadien-1-ol (106-25-2) |  |
| --- | --- |
| LD50 oral rat | 4500 mg/kg Source: IUCLID,NLM,THOMSON |
| LD50 oral | 4500 mg/kg |
| LD50 dermal rabbit | > 5000 mg/kg Source: IUCLID,NLM,THOMSON |
| 3,7-Dimethyl octa-1,6-diene-3-yl acetate (115- | 95-7) |
| LD50 oral rat | 13934 mg/kg Source: HSDB |
| LD50 dermal rabbit | > 5000 mg/kg Source: HSDB |
| LC50 Inhalation - Rat (Dust/Mist) | > 2.74 mg/l Source: SIDS |

| 4-tert-Butylcyclohexyl acetate (32210-23-4) |  |
| --- | --- |
| LD50 oral rat | 3370 mg/kg Source: NITE |
| LD50 oral | 3370 mg/kg |
| LD50 dermal rabbit | > 4680 mg/kg Source: NITE |
| (Z)-3-Hexenyl 2-hydroxybenzoate (65405-77-8) |  |
| LD50 oral | 2500 mg/kg |

| 4,7,7-trimethyl-8-oxabicyclo[2.2.2]octane (470-82-6) |  |
| --- | --- |
| LD50 oral rat | 2480 mg/kg Source: NLM; chemIDplus, TOMES;LOLI, RTECS; |
| LD50 oral | 2480 mg/kg |

| 1-(2,6,6-Trimethyl-3-cyclohexenyl)-2-buten-1-one (57378-68-4) |  |
| --- | --- |
| LD50 oral | 1400 mg/kg |

| 3,7-Dimethyloctan-3-ol (78-69-3) |  |
| --- | --- |
| LD50 oral rat | > 8270 mg/kg Source: ECHA |
| LD50 dermal rabbit | > 5000 mg/kg Source: National Library of Medicine |

| Coumarin (91-64-5) |  |
| --- | --- |
| LD50 oral rat | 293 mg/kg Source: National Library of Medicine/Hazardous Substances Data Bank |
| LD50 oral | 500 mg/kg |

| 3-(p-Methoxyphenyl)-2-methylpropionaldehyde (5462-06-6) |  |
| --- | --- |
| LD50 oral | 4500 mg/kg |

| Reaction mass of 3,5-dimethylcyclohex-3-ene-1-carbaldehyde and 2,4-dimethylcyclohex-3-ene-1-carbaldehyde (68039-49- 6) |  |
| --- | --- |
| LD50 oral | 3900 mg/kg |

| 2-Benzylideneoctanal (101-86-0) |  |
| --- | --- |
| LD50 oral rat | 2450 – 3750 mg/kg |
| LD50 oral | 3100 mg/kg |
| LD50 dermal rabbit | > 3000 mg/kg |
| LC50 Inhalation - Rat | 5 mg/kg |
| Undecan-4-olide (104-67-6) |  |
| LD50 oral rat | 18500 mg/kg Source: NLM;ChemIDplus, TOMES;LOLI; |

| 2-tert-Butylcyclohexyl acetate (88-41-5) |  |
| --- | --- |
| LD50 oral rat | 4600 mg/kg Source: Registry of Toxic Effects of Chemical Substances¦ |
| LD50 oral | 4600 mg/kg |
| LD50 dermal rabbit | 5000 mg/kg Source: Registry of Toxic Effects of Chemical Substances¦ |

| 3,7-Dimethyl octa-1,6-diene-3-ol (78-70-6) |  |
| --- | --- |
| LD50 oral rat | 3000 mg/kg Source: OECD Screening Information Data Set |
| LD50 oral | 2790 mg/kg |
| LD50 dermal rabbit | ≥ 2000 mg/kg Source: OECD Screening Information Data Set |

| Tetrahydro-4-methyl-2-(2-methylpropyl)-2H-pyran-4-ol (63500-71-0) |  |
| --- | --- |
| LD50 oral rat | > 2000 mg/kg Source: ECHA |
| LD50 dermal rat | > 2000 mg/kg Source: HSDB |

| benzyl salicylate (118-58-1) |  |
| --- | --- |
| LD50 oral rat | > 3000 mg/kg Source: ECHA |
| LD50 oral | 2200 mg/kg |
| LD50 dermal rat | > 2000 mg/kg Source: ECHA |
| Benzyl Acetate (140-11-4) |  |
| LD50 oral rat | 2490 mg/kg |
| LD50 oral | 2490 mg/kg |
| LD50 dermal rabbit | 5000 mg/kg |
| 3-ethoxy-4-hydroxybenzaldehyde (121-32-4) |  |
| LD50 oral rat | 1590 mg/kg Source: NLM, THOMSON |
| LD50 oral | 3000 mg/kg |
| LD50 dermal rabbit | > 7940 mg/kg Source: NLM |

| Skin corrosion/irritation | : Not classified |
| --- | --- |
| Serious eye damage/irritation | : Causes serious eye irritation. |
| Respiratory or skin sensitisation | : May cause an allergic skin reaction. |
| Germ cell mutagenicity | : Not classified |
| Carcinogenicity | : Not classified |

| Coumarin (91-64-5) |  |  |
| --- | --- | --- |
| IARC group |  | 3 - Not classifiable |
| Benzyl Acetate (140-11-4) |  |  |
| IARC group |  | 3 - Not classifiable |
| Reproductive toxicity | : | Not classified |
| STOT-single exposure | : | Not classified |
| STOT-repeated exposure | : | Not classified |
| Aspiration hazard | : | Not classified |
| 11.2. Information on other hazards |  |  |
| No additional information available |  |  |
| SECTION 12: Ecological information |  |  |
| 12.1. Toxicity |  |  |
| Ecology - general | : | Harmful to aquatic life with long lasting effects. |
| Hazardous to the aquatic environment, short–term (acute) | : | Not classified |
| Hazardous to the aquatic environment, long–term (chronic) | : | Harmful to aquatic life with long lasting effects. |
| 12.2. Persistence and degradability |  |  |
| JASMIN AND ROSE DE MAI 1644370 |  |  |
| Persistence and degradability |  | Not rapidly degradable |

| geraniol; (2E)-3,7-dimethylocta-2,6-dien-1-ol (106-24-1) |  |
| --- | --- |
| Persistence and degradability | Not rapidly degradable |

| cis-3,7-Dimethyl-2,6-octadien-1-ol (106-25-2) |  |
| --- | --- |
| Persistence and degradability | Not rapidly degradable |
| 3,7-Dimethyl octa-1,6-diene-3-yl acetate (115- | 95-7) |
| Persistence and degradability | Not rapidly degradable |

| 3-Methyl-4-(2,6,6-trimethyl-2-cyclohexenyl)-3-buten-2-one (127-51-5) |  |
| --- | --- |
| Persistence and degradability | Not rapidly degradable |

| 4-tert-Butylcyclohexyl acetate (32210-23-4) |  |
| --- | --- |
| Persistence and degradability | Not rapidly degradable |
| (1637294-12-2) |  |
| Persistence and degradability | Not rapidly degradable |

| (Z)-3-Hexenyl 2-hydroxybenzoate (65405-77-8) |  |
| --- | --- |
| Persistence and degradability | Not rapidly degradable |
| 4,7,7-trimethyl-8-oxabicyclo[2.2.2]octane (470 | -82-6) |
| Persistence and degradability | Not rapidly degradable |

| 1-(2,6,6-Trimethyl-3-cyclohexenyl)-2-buten-1-one (57378-68-4) |  |
| --- | --- |
| Persistence and degradability Not rapidly degradable |  |
| 3,7-Dimethyloctan-3-ol (78-69-3) |  |
| Persistence and degradability | Not rapidly degradable |

| Coumarin (91-64-5) |  |
| --- | --- |
| Persistence and degradability | Not rapidly degradable |

| 1-(1,2,3,4,5,6,7,8-Octahydro-2,3,8,8-tetramethyl-2-naphthyl)ethan-1-one (54464-57-2) |  |
| --- | --- |
| Persistence and degradability | Not rapidly degradable |
| 3-(p-Methoxyphenyl)-2-methylpropionaldehyd | e (5462-06-6) |
| Persistence and degradability | Not rapidly degradable |

| Reaction mass of 3,5-dimethylcyclohex-3-ene-1-carbaldehyde and 2,4-dimethylcyclohex-3-ene-1-carbaldehyde (68039-49- 6) |  |
| --- | --- |
| Persistence and degradability | Not rapidly degradable |

| Oxacyclohexadecen-2-one (34902-57-3) |  |
| --- | --- |
| Persistence and degradability | Not rapidly degradable |
| 2-Benzylideneoctanal (101-86-0) |  |
| Persistence and degradability | Not rapidly degradable |

| Undecan-4-olide (104-67-6) |  |
| --- | --- |
| Persistence and degradability | Not rapidly degradable |
| 2-tert-Butylcyclohexyl acetate (88-41-5) |  |
| Persistence and degradability | Not rapidly degradable |

| 3,7-Dimethyl octa-1,6-diene-3-ol (78-70-6) |  |
| --- | --- |
| Persistence and degradability | Not rapidly degradable |

| Tetrahydro-4-methyl-2-(2-methylpropyl)-2H-pyran-4-ol (63500-71-0) |  |
| --- | --- |
| Persistence and degradability | Not rapidly degradable |
| benzyl salicylate (118-58-1) |  |
| Persistence and degradability | Not rapidly degradable |

| Benzyl Acetate (140-11-4) |  |
| --- | --- |
| Persistence and degradability | Not rapidly degradable |

| 3-ethoxy-4-hydroxybenzaldehyde (121-32-4) |  |
| --- | --- |
| Persistence and degradability | Not rapidly degradable |

## 12.3. Bioaccumulative potential

| geraniol; (2E)-3,7-dimethylocta-2,6-dien-1-ol (106-24-1) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 3.56 |

| cis-3,7-Dimethyl-2,6-octadien-1-ol (106-25-2) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 3.47 |
| 3,7-Dimethyl octa-1,6-diene-3-yl acetate (115- | 95-7) |
| Partition coefficient n-octanol/water (Log Pow) | 3.93 Source: NLM;ChemIDPlus |

| 3-Methyl-4-(2,6,6-trimethyl-2-cyclohexenyl)-3-buten-2-one (127-51-5) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 4.7 Source: ECHA Registered substances |

| 4-tert-Butylcyclohexyl acetate (32210-23-4) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 4.8 Source: Pubchem |
| 4,7,7-trimethyl-8-oxabicyclo[2.2.2]octane (470 | -82-6) |
| Partition coefficient n-octanol/water (Log Pow) | 2.74 |

| 3,7-Dimethyloctan-3-ol (78-69-3) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 3.6 Source: Ecological Structure Activity Relationships |
| Coumarin (91-64-5) |  |
| Partition coefficient n-octanol/water (Log Pow) | 1.39 Source: International Chemical Safety Cards |

| 1-(1,2,3,4,5,6,7,8-Octahydro-2,3,8,8-tetramethyl-2-naphthyl)ethan-1-one (54464-57-2) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 5.18 Source: Episuite |
| 2-Benzylideneoctanal (101-86-0) |  |
| Partition coefficient n-octanol/water (Log Pow) | 4.82 Source: EPI SUITE |

| Undecan-4-olide (104-67-6) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 3.06 Source: NITE |
| 2-tert-Butylcyclohexyl acetate (88-41-5) |  |
| Partition coefficient n-octanol/water (Log Pow) | 4.42 Source: Quantitative Structure Activity Relation |

| 3,7-Dimethyl octa-1,6-diene-3-ol (78-70-6) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 2.97 Source: International Chemical Safety Cards |

| Tetrahydro-4-methyl-2-(2-methylpropyl)-2H-pyran-4-ol (63500-71-0) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 1.65 Source: ECHA |
| benzyl salicylate (118-58-1) |  |
| Partition coefficient n-octanol/water (Log Pow) | 4 Source: ECHA Registered substances |

| Benzyl Acetate (140-11-4) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 1.96 |

| 3-ethoxy-4-hydroxybenzaldehyde (121-32-4) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 1.58 Source: ChemIDplus |

## 12.4. Mobility in soil

| 3,7-Dimethyl octa-1,6-diene-3-yl acetate (115-95-7) |  |
| --- | --- |
| Mobility in soil | 432.4 Source: EPISUITE |

| 4,7,7-trimethyl-8-oxabicyclo[2.2.2]octane (470-82-6) |  |
| --- | --- |
| Mobility in soil | 223.9 Source: EPISUITE |
| 3,7-Dimethyloctan-3-ol (78-69-3) |  |
| Mobility in soil | 319.8 Source: EPI Suite |

| Coumarin (91-64-5) |  |
| --- | --- |
| Mobility in soil | 140 Source: National Library of Medicine/Hazardous Substances Data Bank |

| Reaction mass of 3,5-dimethylcyclohex-3-ene-1-carbaldehyde and 2,4-dimethylcyclohex-3-ene-1-carbaldehyde (68039-49- 6) |  |
| --- | --- |
| Mobility in soil | 187.2 Source: EPISUITE v4.1 |

| 2-Benzylideneoctanal (101-86-0) |  |
| --- | --- |
| Mobility in soil | 2301 Source: EPI SUITE |
| Undecan-4-olide (104-67-6) |  |
| Mobility in soil | 356.5 |

| 2-tert-Butylcyclohexyl acetate (88-41-5) |  |
| --- | --- |
| Mobility in soil | 2015 Source: EPI Suite |
| 3,7-Dimethyl octa-1,6-diene-3-ol (78-70-6) |  |
| Mobility in soil | 76 Source: HSDB |

12.5. Results of PBT and vPvB assessment

No additional information available

12.6. Endocrine disrupting properties

No additional information available

## 12.7. Other adverse effects

No additional information available

| SECTION 13: Disposal considerations |  |  |
| --- | --- | --- |
| 13.1. Waste treatment methods |  |  |
| Regional waste regulation | : | Disposal must be done according to official regulations. |
| Waste treatment methods | : | Dispose of contents/container in accordance with licensed collector’s sorting instructions. |
| Sewage disposal recommendations | : | Disposal must be done according to official regulations. |
| Product/Packaging disposal recommendations | : | Disposal must be done according to official regulations. |
| Additional information | : | Do not re-use empty containers. |
| SECTION 14: Transport information |  |  |

In accordance with ADR / IMDG / IATA / ADN / RID

| ADR | IMDG | IATA | ADN | RID |
| --- | --- | --- | --- | --- |
| 14.1. UN number or ID number |  |  |  |  |
| Not applicable | Not applicable | Not applicable | Not applicable | Not applicable |
| 14.2. UN proper shipping name |  |  |  |  |
| Not applicable | Not applicable | Not applicable | Not applicable | Not applicable |
| 14.3. Transport hazard class(es) |  |  |  |  |
| Not applicable | Not applicable | Not applicable | Not applicable | Not applicable |
| 14.4. Packing group |  |  |  |  |
| Not applicable | Not applicable | Not applicable | Not applicable | Not applicable |
| 14.5. Environmental hazards |  |  |  |  |
| Not applicable | Not applicable | Not applicable | Not applicable | Not applicable |
| No supplementary information available |  |  |  |  |

## 14.6. Special precautions for user

Overland transport

Not applicable

Transport by sea

Not applicable

Air transport

Not applicable

Inland waterway transport

Not applicable

Rail transport

Not applicable

14.7. Maritime transport in bulk according to IMO instruments

Not applicable

# SECTION 15: Regulatory information

## 15.1. Safety, health and environmental regulations/legislation specific for the substance or mixture

EU-Regulations REACH Annex XVII (Restriction List)

Contains no substance(s) listed on REACH Annex XVII (Restriction Conditions)

REACH Annex XIV (Authorisation List)

Contains no substance(s) listed on REACH Annex XIV (Authorisation List)

REACH Candidate List (SVHC)

Contains no substance(s) listed on the REACH Candidate List

PIC Regulation (Prior Informed Consent)

Contains no substance(s) listed on the PIC list (Regulation EU 649/2012 concerning the export and import of hazardous chemicals)

POP Regulation (Persistent Organic Pollutants)

Contains no substance(s) listed on the POP list (Regulation EU 2019/1021 on persistent organic pollutants)

Ozone Regulation (1005/2009)

Contains no substance(s) listed on the Ozone Depletion list (Regulation EU 1005/2009 on substances that deplete the ozone layer)

### Dual-Use Regulation (428/2009)

Contains no substance subject to the COUNCIL REGULATION (EC) No 428/2009 of 5 May 2009 setting up a Community regime for the control of exports, transfer, brokering and transit of dual-use items.

Explosives Precursors Regulation (2019/1148)

Contains no substance(s) listed on the Explosives Precursors list (Regulation EU 2019/1148 on the marketing and use of explosives precursors)

### Drug Precursors Regulation (273/2004)

Contains no substance(s) listed on the Drug Precursors list (Regulation EC 273/2004 on the manufacture and the placing on market of certain substances used in the illicit manufacture of narcotic drugs and psychotropic substances)

15.2. Chemical safety assessment

No chemical safety assessment has been carried out

# SECTION 16: Other information

| Abbreviations and acronyms: |  |
| --- | --- |
| ADN | European Agreement concerning the International Carriage of Dangerous Goods by Inland Waterways |
| ADR | European Agreement concerning the International Carriage of Dangerous Goods by Road |
| ATE | Acute Toxicity Estimate |
| BCF | Bioconcentration factor |
| BLV | Biological limit value |
| BOD | Biochemical oxygen demand (BOD) |
| COD | Chemical oxygen demand (COD) |
| DMEL | Derived Minimal Effect level |
| DNEL | Derived-No Effect Level |
| EC-No. | European Community number |
| EC50 | Median effective concentration |
| EN | European Standard |
| IARC | International Agency for Research on Cancer |
| IATA | International Air Transport Association |
| IMDG | International Maritime Dangerous Goods |
| LC50 | Median lethal concentration |
| LD50 | Median lethal dose |
| LOAEL | Lowest Observed Adverse Effect Level |
| NOAEC | No-Observed Adverse Effect Concentration |
| Abbreviations and acronyms: |  |
| NOAEL | No-Observed Adverse Effect Level |
| NOEC | No-Observed Effect Concentration |
| OECD | Organisation for Economic Co-operation and Development |
| OEL | Occupational Exposure Limit |
| PBT | Persistent Bioaccumulative Toxic |
| PNEC | Predicted No-Effect Concentration |
| RID | Regulations concerning the International Carriage of Dangerous Goods by Rail |
| SDS | Safety Data Sheet |
| STP | Sewage treatment plant |
| ThOD | Theoretical oxygen demand (ThOD) |
| TLM | Median Tolerance Limit |
| VOC | Volatile Organic Compounds |
| CAS-No. | Chemical Abstract Service number |
| N.O.S. | Not Otherwise Specified |
| vPvB | Very Persistent and Very Bioaccumulative |
| ED | Endocrine disruptor |

| Full text of H- and EUH-statements: |  |
| --- | --- |
| Acute Tox. 4 (Inhalation) | Acute toxicity (inhal.), Category 4 |
| Acute Tox. 4 (Oral) | Acute toxicity (oral), Category 4 |
| Aquatic Acute 1 | Hazardous to the aquatic environment – Acute Hazard, Category 1 |
| Aquatic Chronic 1 | Hazardous to the aquatic environment – Chronic Hazard, Category 1 |
| Aquatic Chronic 2 | Hazardous to the aquatic environment – Chronic Hazard, Category 2 |
| Aquatic Chronic 3 | Hazardous to the aquatic environment – Chronic Hazard, Category 3 |
| EUH208 | Contains Pentadecan-15-olide. May produce an allergic reaction. |
| Eye Dam. 1 | Serious eye damage/eye irritation, Category 1 |
| Eye Irrit. 2 | Serious eye damage/eye irritation, Category 2 |
| Flam. Liq. 3 | Flammable liquids, Category 3 |
| H226 | Flammable liquid and vapour. |
| H302 | Harmful if swallowed. |
| H315 | Causes skin irritation. |
| H317 | May cause an allergic skin reaction. |
| H318 | Causes serious eye damage. |
| H319 | Causes serious eye irritation. |
| H332 | Harmful if inhaled. |
| H400 | Very toxic to aquatic life. |
| H410 | Very toxic to aquatic life with long lasting effects. |
| H411 | Toxic to aquatic life with long lasting effects. |
| H412 | Harmful to aquatic life with long lasting effects. |
| Full text of H- and EUH-statements: |  |
| Skin Irrit. 2 | Skin corrosion/irritation, Category 2 |
| Skin Sens. 1 | Skin sensitisation, Category 1 |
| Skin Sens. 1A | Skin sensitisation, category 1A |
| Skin Sens. 1B | Skin sensitisation, category 1B |

| Classification and procedure used to derive the classification for mixtures according to Regulation (EC) 1272/2008 [CLP]: |  |  |
| --- | --- | --- |
| Eye Irrit. 2 | H319 | Calculation method |
| Skin Sens. 1 | H317 | Calculation method |
| Aquatic Chronic 3 | H412 | Calculation method |

The classification complies with 	: ATP 12

This information is based on our current knowledge and is intended to describe the product for the purposes of health, safety and environmental requirements only. It should not therefore be construed as guaranteeing any specific property of the product.